User’s Manual

USB TO RS-232 CONVERTER

# Overview

Universal Serial Bus (USB) port technology has emerged

in response to the proliferation of external peripheral devices

(scanner, digital camera, removable drives ,etc.) that are

increasingly being connected to the latest generation of

PCs. The USB behaves in a similar fashion to conventional

bus ports(i.e,serial,parallel,PS/2),but since it does not

require any IRQs, more devices can be attached to the

System, any the problem of IRQ conflict is eliminated.

The USB Serial adaptor provides an external plug-and

-play RS-232 modem connection for PC’s, Notebooks,

Laptops, and handheld computing devices that support

the USB specification. The adaptor comes with a

standard DB-25 Male/DB-9 Male connector to plug into

the Rs232 port, and has a 1.8m USB cable with a type A

plug for connection to the PC’s USB port or to a USB hub.

Note: Since most computers only have one or two USB

ports, USB hubs are often used to provide additional                     	ports so that numerous USB peripheral devices can

be connected up at the same time.

For further convenience-especially to users of notebooks,

laptops. And handholds – the USB to RS-232 Adaptor

device its operating power from the Universal Serial Bus

itself, so no bulky external power supply needs to be taken

along. And because it is a USB Specification compliant

device, Supports energy-saving suspend and resume

operations.

## System Requirements

- Windows 98/ME/XP/2000,Mac8.6 and higher, Linux.

- USB Host Controller installed on the system.

## Features

- Supports automatic handshake mode.

- Works with cellular phones, PDA, Digital cameras,

Modems, and ISDN terminal adapters.

- Supports remote wake-up and power management.

- Frees your RS-232 Ports for other uses.

Adapter Installation:

1.Plug the Adaptor into the computer’s USB port; the NEW

Hardware Wizard appears; click on Next to continue.

2.In the dialog box that comes up, leave the default choice

(Search for the best drives for your device): Click on

Next to continue

3.In the dialog box that comes up, select the proper drive

for your media; insert the distribution diskette in the drive

drive; Click on Next to continue

4.In the Ready to install…dialog box that comes up; Click

on Next to continue ,Files are now copied to your hard disk

5.After the driver files have been copied to the hard disk, a

dialog box appears to tell you that the installation has

finished; Click on Finish. Windows now finishes up the

installation.

TO check that the adapter was correctly installed:

1.Open the My Computer folder.

2.Open the Control Panel folder.

3.Open the System folder

4.Click on the Device Manager tab at the top of the dialog

Box.

5.Click on the plus sign in front of the Universal Serial Bus

Controller heading to see the Universal Serial Bus

Controller listing.

If the installation completed successfully, you should see

an entry for USB Serial Port in the listing.

Modem Installation:

Note: This procedure is only necessary if you have never

connected up your modem before. If your modem was

previously connected and configured for the RS-232

(COM)port after you perform the Adaptor Installation

and click on Finish, the driver will detect your modem

and install it automatically, so You can skip this section.

1.Plug the Adaptor into the modem; then turn the modem On.

2.Open the My Computer folder.

3.Double click on Modems the install a NEW Modem.

dialog box appears.

4.Click on Next to continue.

5.After Windows detects your modem (this procedure can

take a few minutes).the Verify Modem dialog box appears

Click on Next to continue.

6.Fill in the Location Information dialog box, then chick on

Next to continue.

7.A dialog box appears to tell you that the installation has finished. Click on Finish to complete the installation.

Method 2(If the modem has previously been connected to

the RS-232 port):

Specifications

| Function |  | UMC-100 | UMC-101 | UMC-201 | UMC-202 |
| --- | --- | --- | --- | --- | --- |
| Chipset |  | FTDI |  |  |  |
| Complies USB Rev. |  | Rev 1.1 |  |  |  |
| Connector | Input | USB B Type/F | USB TYPE/F | USB A TYPE /M | USB B TYPE/F |
|  | Output | DB-25/M | DB-25/M+DB-9M | DB-9/M |  |
| Power Mode |  | Bus |  |  |  |
| Cable Length |  | 1.8M | 0.4M | 1M | 1.8M |
| Housing |  | Plastic |  |  |  |